EE2003 – Computer Organization and Assembly Language (Fall 2025)                                    09 Sep 2025
Quiz: 01; Weight: 2.0; Time: 15 Minutes; Marks: 10;
Instructor: Arshi Parvaiz

Name: _______________________________________     Roll No. P ___- _______   Section: BCS-3C

Note: Direct answers without steps are not acceptable.

Q1: A computer system is designed with a memory capacity of 2 GB and uses an address bus with 32 address lines. What is the required width of the data bus (in bits) for this system to fully utilize the memory?  (Also, show your work)	.		   [5 Marks]						                   
Ans:

The number of addresses with 32 lines are 232=4,294,967,296. ≈ 4G. If the system is byte-addressable, then each address corresponds to 1 byte. So with 32 address lines, the maximum addressable memory =

232×1 byte=4 GB

The Required memory is 2 GB. To make the total capacity exactly 2 GB, each address should map to less than 1 byte. 232×1/2 byte= 2 GB.  So data bus is 4bits.

Q2: Consider the following assembly snippet:                                                               [5 Marks]

mov ax, [num1]   ; Instruction A

mov bx, [num1+1] ; Instruction B

num1: db 5, 10, 15, 0

- Identify and explain the size mismatch in Instructions.

- If the programmer wants to correctly add all four numbers, rewrite the code using appropriate registers and instructions.

Ans-

- num1 is defined with db, i.e., bytes (8-bit values). But mov ax, [num1] and mov bx, [num1+1] are word (16-bit) operations. This creates a size mismatch: the data is byte-sized, but the instructions are trying to load words. For example, mov ax, [num1] would read two bytes (05h and 0Ah) as a single 16-bit word, which is not what the programmer intended.

- mov al, [num1]      ; AL = 5

mov bl, [num1+1]    ; BL = 10

add al, bl          ; AL = 15

mov bl, [num1+2]    ; BL = 15

add al, bl          ; AL = 30

mov bl, [num1+3]    ; BL = 0

add al, bl          ; AL = 30