EE2003 – Computer Organization and Assembly Language (Fall 2025)                                    09 Sep 2025
Quiz: 01; Weight: 2.0; Time: 15 Minutes; Marks: 10;
Instructor: Arshi Parvaiz

Name: _______________________________________     Roll No. P ___- _______   Section: BCS-3C

Note: Direct answers without steps are not acceptable.

Q1: A computer system has a memory capacity of 4 GB and uses an address bus with 30 address lines and a data bus with a width of 16 bits. Will this system be able to address the entire 4 GB of memory fully? Justify your answer. 			           		    [5 Marks]

Ans:

The system has 30 address lines, so it can generate 230 addresses. With a 16-bit (2-byte) data bus, the total addressable capacity is 2×230= 2GB. Since the memory capacity is 4 GB, this system cannot fully address the entire memory.

Q2: Consider the following assembly snippet:

mov al, [num1]   ; Instruction A

num1: dw 1234h

- a. Identify and explain the problem in Instruction A with respect to how data is defined at num1.
b. Rewrite the code correctly if the programmer wants to:

  - Load only the lower byte (34h) into a register.

  - Load the full word (1234h) into a register.

  - Load the higher byte (12h) into a register.                                               	[5 Marks]

Ans:

- The directive dw 1234h defines a word (16 bits = 2 bytes) at num1. Because of the little endian, at offset num1, the lower byte (34h) is stored. And at offset num1+1, the higher byte (12h) is stored.
But Instruction A (mov al, [num1]) moves only one byte (34h) into AL.
So there is a mismatch: data was defined as a word, but the instruction is accessing only a byte.

- Mov al, [num1]

- Mov ah, [num1+1]

- Mov bx, [num1]