DLD  Lab-06

Subtractor

NATIONAL UNIVERSTIY OF COMPUTER AND EMERGING SCIENCES, FAST- Peshawar Campus

Department Of Computer Science

Instructor: Engr. Waseem Ullah

EL1005 – Digital Logic Design-Lab

SEMESTER SPRING 2025

Contents

1.	Objectives:	2

2.	Equipment Required:	2

3.	What is Subtractor ?	2

4.	Half  subtractor	2

5.	FULL  Subtractor	3

6.	K-Map Minimization	4

7.	Applications	5

# Objectives:

To become familiar with the operation of adders and Subtractors

# Equipment Required:

- DEV-2765E Trainer Board/ Multisim 14.2 /Logic.ly

- 7486 quad 2-input XOR gate IC

- 7404 Hex Inverter gate IC

- 7408 quad 2-input AND gate IC

- 7432 quad 2-input OR gate IC

# What is Subtractor ?

Subtractor is an electronic logic circuit  for calculating the difference between  two binary numbers which provides the  difference and borrow as output.

# Half  subtractor

Half Subtractor is used for subtracting one single bit binary number from another single bit binary number.It has two inputs; Minuend (A) and  Subtrahend (B) and two outputs; Difference (D) and Borrow (Bout).

Difference = A ⊕ B

Difference = A ⊕ B

Truth Table

| Input |  | Output |  |
| --- | --- | --- | --- |
| A | B | Difference (D) | Borrow (Bout) |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |

From the truth table and K-map, the Boolean  Expression can be derived as:

Difference (D) = A'.B + A.B'

A'.B + A.B' =	A ⊕ B

Borrow (Bout)=	Ā.B

# FULL  Subtractor

A logic Circuit Which is used for subtracting  three single bit binary numbers is known as  Full Subtractor.

It has three inputs; Minuend (A), Subtrahend

(B) and following Subtrahend (C) and two  outputs; Difference (D) and Borrow (Bout).

- Truth Table

| Input |  |  | Output |  |
| --- | --- | --- | --- | --- |
| A | B | B(in) | D | B(out) |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

- Solving Truth Table using K-Map

# K-Map Minimization

From the Truth Table The Difference and Borrow will  written as,

Difference=A'B'C+A'BC'+AB'C'+ABC

Reducing it we got,  Difference=A ⊕B ⊕C

Borrow=A'B'C+A'BC'+A'BC+ABC

=A'B'C+A'BC'+A'BC+A'BC+A'BC+ABC

=A'C(B'+B)+A'B(C'+C)+BC(A'+A)  Borrow=A'C+A'B+BC

From the truth table and k-map minimization,  the Boolean Expression can be derived as:

D	= A ⊕ B ⊕ C

B(out) = BC + (B ⊕ C) A

# Applications

- To attenuate the radio/audio signal

- In amplifier to reduce sound distortion

- In arithmetic logic unit of processors

- Increment and decrement operators

- Calculate addresses

# Lab Task

- Design and implement the circuitry for a Half subtractor.

- Implement a half subtractor using AND, OR and NOT gates only.

- Implement a half subtractor using IC-74LS139.

- Design and implement the circuitry of a full subtractor.