LAB 8

Task 1:

| Code | Explanation |
| --- | --- |
| [org 0x0100] jmp start num: db 11100101b start: mov al, num rol al, 1 ror al, 1 mov ax, 0x4c00 int 0x21 | - Start. - saved 8 bit number 11100101b - moved the number to al register - rotate left result (11001010b) CF = 1. - Rotate right result the same number - Exit program |

Task 2:

| Code | Explanation |
| --- | --- |
| [org 0x0100] jmp start multiplicand: db 4 multiplier: db 3 result: db 0 start: mov cl, 4 mov bl, [multiplicand] mov dl, [multiplier] checkbit: shr dl, 1 jnc skip add [result], bl skip: shl bl, 1 dec cl jnz checkbit mov ax, 0x4c00 int 0x21 | - Start. - store 4 in multipicand label - store 3 in multiplier label - store 0 in result label. - mov 4 in cl register - mov the value of multiplicand into bl register which is 4. - mov the value of multiplier into dl register which is 3. - run loop 4 times. - each time shift right the value of dl by 1. - check if carry is 0 if it is then jump to skip label else add value of bl into result. - then shift left bl by 1 - repeat the process |

Task 3:

| Code | Explanation |
| --- | --- |
| [org 0x0100] jmp start num1: dw 000Ah num2: dw 0003h high: dw 0000h low: dw 0000h num3: dw 00A0h num4: dw 000Ah s_high: dw 0000h s_low: dw 0000h start: clc mov ax, [num1] add ax, [num2] mov word [low], ax adc word [high], 0 mov ax, [num3] sub ax, [num4] mov word [s_low], ax sbb word [s_high], 0 mov ax, 0x4c00 int 0x21 | - Initialize some important labels for addtion and subtraction...num1, num2 , high and low for extended addition. - num3, num4, s_high and s_low for extended subtraction. - after start label first clear the carry to 0. - mov value of num1 into ax register. - add value of num2 into ax register. - mov value of qx into low. - add carry into high For subtraction: - mov num3 into ax - subtract num3 from ax and store into ax. - mov value of ax into s_low. - subtract bit - end program |

Task 4:

| Code | Explantion |
| --- | --- |
| [org 0x0100] jmp start num1: db 00A0h num2: db 00BBh result: db 0 start: mov al, [num1] mov bl, [num2] and al, bl mov [result], al mov al, [num1] or al, bl mov [result], al mov al, [num1] xor al, bl mov [result], al mov al, [num1] not al mov [result], al mov ax, 0x4c00 int 0x21 | - start - store some values in num1 and num2. - result = 0. - store num1 in al register and num2 in bl register. - Perform AND operation. give 1 only when both bits are 1. mov the result into result label. - Now again mov num1 into al register. - Perform OR operation, gives 1 when either al bit is 1 or bl bit is 1. - mov result into result. - again mov num1 into al. - Perform xor operation. gives 1 only if both values are different. - store result into result. - mov num1 into al. - perform not operation into al - mov result into result |